DAM MASTER AI

Calculation Handbook & Validation Record

Every formula, rule and method used by the software, with variables, worked examples, references and test results

| Item | Detail |
| --- | --- |
| Document ID | RCPL-DMAI-KB-001 |
| Software version | DAM MASTER AI Platform v1.0 |
| Date | 6 October 2026 |
| Entries | 47 formulas and rules |
| Validation | 37 of 37 automatic tests passed |
| Prepared by | Digital vertical, Rodic Hub — Rodic Consultants Pvt. Ltd. |

# Contents

Right-click and choose "Update field" to show page numbers.

# 1. How to use this handbook

This handbook is the printed form of the Knowledge Base inside the software. Every number on screen with an "i" button links to one of the entries below, and the same entry is shown with the user's actual values substituted.

- Formula — the exact expression the software evaluates.

- Variables — symbols, meaning and units.

- Method, assumptions and criteria — how it is applied and its limits.

- Worked example — numbers that can be checked by hand.

- References — the governing standard or guidance. Editions and clauses are entered and verified by a named engineer in the software (References & code validation); this handbook does not assert clause numbers.

Status "Executed in system" means the software runs the formula; "Reference (not executed)" means it is documented for engineers but values are imported already converted.

All design outputs are preliminary engineering for option comparison. Detailed design, verification against the governing editions of the codes and approval by authorised engineers are required.

# 2. Data import & quality

## 2.1 Row rejection, quarantine and duplicates

Used in: Import Excel → validation   ·   Executed in system

Decides whether an imported row is saved at all.

### Formula

Reject if any of: dam_id, sensor_id, parameter_code, observed_at, value, unit is blank

Reject if value is not a number

Quarantine if sensor_id is not in Sensor_Master (file or system)

Reject if dam_id or parameter_code differs from the sensor master

Reject if unit ≠ sensor eng_unit (case-insensitive) — QC-07

Reject if observed_at cannot be read, or is more than 5 minutes in the future — QC-04

Skip if the same sensor already has a reading at the same observed_at (stored or earlier in file) — FR-ING-03

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| observed_at | Measurement time, IST (UTC+05:30) | date-time |

### Method

Rules applied in the order listed; the first failing rule decides the outcome.

### Assumptions and limits

- Excel serial dates are interpreted as IST wall-clock time

- Text dates accepted: yyyy-mm-dd hh:mm, dd-mm-yyyy hh:mm, dd/mm/yyyy hh:mm, ISO 8601 with offset

### Acceptance criteria

- Every rejected / quarantined / skipped row is listed with its rule in the issue report

### Worked example

Inputs: Row: sensor DM001-PZ-07, unit 'psi', sensor unit 'kPa'

Unit check: 'psi' ≠ 'kPa'

Result: Rejected — QC-07 unit

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §5–6 | Project requirement |

## 2.2 QC-01 Range check and QC-05 logger status

Used in: Import → quality flag B   ·   Executed in system

Catches physically impossible readings and logger faults.

### Formula

Flag B if value < expected_min or value > expected_max

Flag B if logger_status ∉ {OK, MANUAL}

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| expected_min / expected_max | Plausible physical limits from Sensor_Master | sensor unit |

### Method

Applied per row before saving.

### Assumptions and limits

- Limits are set by the instrumentation engineer, not by the system

### Acceptance criteria

- Flag B readings are stored but excluded from AI analysis and the DHI

### Worked example

Inputs: SPF reading 950 L/min, expected_max 400

950 > 400

Result: Saved with flag B (QC-01)

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §6.2 | Project requirement |

## 2.3 QC-02 Isolated spike

Used in: Import → quality flag S   ·   Executed in system

Marks a single reading that jumps away and straight back — usually electrical noise.

### Formula

dₖ = |vₖ − vₖ₋₁| for consecutive non-B readings

m = median of the previous up to 30 values of dₖ (needs ≥ 10)

Flag S if |vₖ − vₖ₋₁| > 5·m AND (no next reading OR next change is opposite in sign and > 2.5·m)

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| vₖ | Reading k (time-ordered) | sensor unit |
| m | Median absolute step | sensor unit |

### Method

Recomputed on the merged series (stored + new) every import.

### Assumptions and limits

- A sustained step change is NOT flagged (it may be real)

### Acceptance criteria

- Flag S readings are still used by AI, with lower confidence

### Worked example

Inputs: Series …100, 101, 100, 125, 100… with median step 1.0

Jump +25 > 5 × 1.0

Next change −25: opposite sign, 25 > 2.5

Result: 125 flagged S (QC-02)

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §6.2 | Project requirement |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Isolated spike flagged once | 1 | 1 | PASS |

## 2.4 QC-03 Flatline

Used in: Import → quality flag S   ·   Executed in system

Detects a stuck sensor or logger repeating the same value.

### Formula

Flag S on every reading in a run of ≥ 12 consecutive identical non-B values

### Method

Not applied to rainfall, acceleration, gate opening and air temperature (event / cumulative data).

### Acceptance criteria

- 12 readings = 3 days at a 6-hour interval

### Worked example

Inputs: 14 consecutive readings of 9.82 mm

Run length 14 ≥ 12

Result: All 14 flagged S (QC-03)

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §6.2 | Project requirement |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Flatline flags 14 identical readings | 14 | 14 | PASS |

## 2.5 Vibrating-wire piezometer conversion (reference)

Used in: Reference — values are imported in engineering units in v1.0   ·   Reference (not executed)

Converts a vibrating-wire reading to pressure when loggers export raw digits.

### Formula

P = G · (R₀ − R₁) + K · (T₁ − T₀)

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| P | Pressure | kPa |
| G | Linear gauge factor (calibration sheet) | kPa/digit |
| R₀, R₁ | Zero reading, current reading | digits |
| K | Thermal factor | kPa/°C |
| T₀, T₁ | Temperature at zero and current reading | °C |

### Method

Linear form; polynomial form A·R² + B·R + C used where the sheet specifies it.

### Assumptions and limits

- Sign convention follows the manufacturer's calibration sheet

### Worked example

Inputs: G = −0.0235 kPa/digit, R₀ = 8650, R₁ = 8235.6, K = 0.1, T₀ = 25, T₁ = 27.1

−0.0235 × (8650 − 8235.6) = −9.74

Sign per sheet convention → see note

Result: Illustrative only — use the certificate

### References

| Document | Scope |
| --- | --- |
| Manufacturer calibration certificate | Governs constants and sign convention |
| IS 7436 — Measurements for structures in river valley projects | Instrument types and practice |

## 2.6 90° V-notch weir discharge (reference)

Used in: Reference — seepage imported in L/min   ·   Reference (not executed)

Converts head over a 90° V-notch to flow.

### Formula

Q = 1.38 · H^2.5   (Q in m³/s, H in m)

Q (L/min) = Q (m³/s) × 60 000

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| H | Head over notch vertex | m |

### Method

Standard thin-plate 90° notch with Cd ≈ 0.58.

### Assumptions and limits

- Free, fully contracted flow; approach velocity negligible

### Worked example

Inputs: H = 0.10 m

1.38 × 0.10^2.5 = 0.004364 m³/s

Result: 261.8 L/min

### References

| Document | Scope |
| --- | --- |
| Weir calibration / site rating | Governs coefficient |

# 3. Monitoring analytics

## 3.1 Anomaly detection — rolling z-score

Used in: Sensor Analysis, Anomalies, Overview   ·   Executed in system

Flags a reading that is unusual compared with the sensor's own last 14 days.

### Formula

Window W = valid readings with t in [tᵢ − 14 days, tᵢ)  (needs ≥ 10)

μ = Σv / n ;  σ = √( Σ(v − μ)² / (n − 1) )

zᵢ = (vᵢ − μ) / σ

Expected band = μ ± z_lim · σ

Anomaly if |zᵢ| > z_lim (default 2.5, set in Rules)

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| vᵢ | Reading tested | sensor unit |
| μ, σ | Window mean and sample standard deviation | sensor unit |
| z_lim | Threshold (rule registry) | – |

### Method

Applied to response instruments only. Not applied to loads (reservoir / tailwater level, inflow) or event data (rainfall, acceleration, gate opening, air temperature).

### Assumptions and limits

- Readings flagged B are excluded; S readings are used

- A steadily trending sensor can produce anomalies at turning points — triage separates noise

### Acceptance criteria

- |z| > 3 counted as significant; |z| > 4 counted in the risk score

### Worked example

Inputs: Window: 10, 12, 11, 13, 12, 11, 10, 12, 13, 11 ; vᵢ = 16

μ = 11.5

σ = 1.0801

z = (16 − 11.5) / 1.0801 = 4.17

Result: Anomaly (|z| = 4.17 > 2.5) → Strong

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §8.3 | Project requirement |
| Statistical process control (standard practice) | Method |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| z-score of a reading | 4.1662 | 4.16619 | PASS |

## 3.2 Anomaly triage

Used in: Anomalies & Alerts, Sensor Analysis   ·   Executed in system

Sorts anomalies by urgency. First matching rule wins.

### Formula

1. v at or beyond alert level → Threshold exceeded (escalate immediately)

2. v within 5% of alert level → Near alert level (escalate)

3. |z| > 4 → Strong anomaly (escalate)

4. |z| > 3 → Moderate anomaly (correlate with level and rain)

5. otherwise → Likely noise (watch)

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| Rising sensor | 'beyond' = v ≥ alert; 'within 5%' = v ≥ alert − 0.05·¦alert¦ |  |
| Falling sensor | 'beyond' = v ≤ alert; 'within 5%' = v ≤ alert + 0.05·¦alert¦ |  |

### Method

Ordered rule list.

### Worked example

Inputs: Rising sensor, alert 215, v = 206, z = 7.0

206 < 215 → rule 1 no

206 ≥ 215 − 10.75 = 204.25 → rule 2 yes

Result: Near alert level

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §8.3 | Project requirement |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Triage — near alert | Near alert level | Near alert level | PASS |
| Triage — strong anomaly | Strong anomaly | Strong anomaly | PASS |

## 3.3 Threshold level of the latest reading

Used in: Overview status, Anomalies, DHI   ·   Executed in system

Compares the latest valid reading with the three threshold levels.

### Formula

Rising: Level 3 if v ≥ emergency; else Level 2 if v ≥ alarm; else Level 1 if v ≥ alert; else Normal

Falling: same with ≤

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| alert / alarm / emergency | Levels from Sensor_Master | sensor unit |

### Method

Evaluated on the latest reading flagged G or S.

### Assumptions and limits

- Level 3 should be confirmed by a repeat reading or second instrument before EAP action (FR-AI-04)

### Worked example

Inputs: SPF 295.2 L/min; alert 290, alarm 340

295.2 ≥ 290

295.2 < 340

Result: Level 1 · Alert

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §8.2 | Project requirement |
| Dam-specific Emergency Action Plan | Defines response per level |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Threshold level | 1 | 1 | PASS |

## 3.4 Trend (least-squares slope)

Used in: Overview trend/day, Sensor forecast   ·   Executed in system

Rate of change of a sensor over the last 21 days.

### Formula

xᵢ = (tᵢ − t_last) / 1 day ;  yᵢ = reading

slope b = Σ(xᵢ − x̄)(yᵢ − ȳ) / Σ(xᵢ − x̄)²

fitted now ŷ₀ = ȳ − b·x̄  (value of the line at the last reading)

forecast ŷ(d) = ŷ₀ + b·d

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| b | Trend | unit per day |
| ŷ₀ | Fitted current value | sensor unit |

### Method

Ordinary least squares; needs ≥ 8 valid readings in 21 days.

### Assumptions and limits

- Linear trend only — production adds the HST model (reservoir, season, time)

### Worked example

Inputs: x = 0,1,2,3,4 ; y = 10, 12, 13, 15, 16

x̄ = 2, ȳ = 13.2

b = 15 / 10 = 1.5

Result: Slope 1.5 per day; ŷ at x̄ = 13.2

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §8.4 | Project requirement |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Least-squares slope | 1.5 | 1.5 | PASS |

## 3.5 Days to alert level

Used in: Overview, Portfolio, Sensor Analysis, Copilot   ·   Executed in system

Estimates how many days until the trend line reaches the alert level.

### Formula

d = (alert − ŷ₀) / b

Shown only if: sensor currently Normal, b moving toward the alert (b > 0 rising, b < 0 falling), and 0 ≤ d ≤ 365

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| ŷ₀, b | From the trend calculation |  |

### Method

Linear extrapolation.

### Assumptions and limits

- Assumes current loading and trend continue

### Worked example

Inputs: Alert 290, ŷ₀ = 272.3, b = 0.89 /day

(290 − 272.3) / 0.89 = 19.9

Result: ≈ 20 days

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §8.4 | Project requirement |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Days to alert | 20 | 20 | PASS |

## 3.6 Correlation with reservoir level

Used in: Sensor Analysis, DHI seepage   ·   Executed in system

Shows whether a sensor moves with the reservoir (expected) or not (investigate).

### Formula

r = Σ(a − ā)(b − b̄) / √( Σ(a − ā)² · Σ(b − b̄)² )

a = sensor readings, b = reservoir level at the same timestamps, last 30 days (needs ≥ 10 pairs)

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| r | Pearson correlation | –1 … +1 |

### Method

Pearson product-moment correlation.

### Assumptions and limits

- Timestamps must match exactly

### Acceptance criteria

- |r| > 0.6 treated as linked to reservoir load

### Worked example

Inputs: a = 1,2,3,4,5 ; b = 2,4,5,4,5

Σ(a−ā)(b−b̄) = 6

√(10 × 6) = 7.746

Result: r = 0.775

### References

| Document | Scope |
| --- | --- |
| Standard statistics | Method |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Pearson correlation | 0.774597 | 0.774597 | PASS |

## 3.7 Data availability and functional sensors

Used in: DHI instrumentation component   ·   Executed in system

Measures how complete the monitoring record is.

### Formula

Expected readings (30 d) = 30 × 1440 / interval_min

Availability % = valid readings in last 30 days ÷ expected × 100 (capped at 100)

Functional = status Active AND last reading within 3 × interval of the data cut-off AND last flag ≠ B

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| interval_min | Expected reading interval | minutes |

### Method

Counts across all non-retired sensors of the dam.

### Worked example

Inputs: 6-hour interval; 116 valid readings in 30 days

Expected = 30 × 1440 / 360 = 120

116 / 120 = 96.7 %

Result: 96.7 %

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §6, §9 | Project requirement |

## 3.8 Seepage–turbidity coincidence (piping signal)

Used in: Overview AI summary   ·   Executed in system

Flags the combination that most often precedes internal erosion.

### Formula

Raise if a seepage-flow anomaly (|z| > 3) and a turbidity anomaly (|z| > 3) occur within 6 hours of each other in the last 30 days

### Method

Rule-based pattern.

### Assumptions and limits

- Advisory — inspection and water sampling confirm

### Worked example

Inputs: SPF anomaly 30-Sep 06:00; TRB anomaly 30-Sep 06:00

Δt = 0 h ≤ 6 h

Result: Possible piping — inspect outfall

### References

| Document | Scope |
| --- | --- |
| Dam safety practice on internal erosion (ICOLD / CWC guidance) | Background |

# 4. Dam Health Index

## 4.1 Dam Health Index (DHI)

Used in: Portfolio, Overview, DHI page, Reports   ·   Executed in system

One 0–100 score for overall dam condition.

### Formula

DHI = 0.25·S + 0.20·Sp + 0.15·I + 0.15·H + 0.15·O + 0.10·D

Bands: Good ≥ 85 · Fair 75–85 · Poor 65–75 · Critical < 65

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| S | Structural condition | 0–100 |
| Sp | Seepage & leakage | 0–100 |
| I | Instrumentation | 0–100 |
| H | Hydrological safety | 0–100 |
| O | O&M / inspection compliance | 0–100 |
| D | Documentation | 0–100 |

### Method

Weighted sum of component scores, each clipped to 0–100.

### Assumptions and limits

- Weights are starting values pending calibration by the dam safety team

### Worked example

Inputs: S 66, Sp 50, I 70, H 74, O 60, D 77

16.5 + 10.0 + 10.5 + 11.1 + 9.0 + 7.7

Result: DHI 64.8 → Critical

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §9 | Project requirement |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| DHI weighted sum | 64.8 | 64.8 | PASS |

## 4.2 DHI — Structural condition

Used in: DHI page   ·   Executed in system

Condition from inspections and deformation instruments.

### Formula

Base = rating score: 5→100, 4→85, 3→70, 2→50, 1→25

− 10 per open High structural finding (max − 30)

− 5 per deformation sensor at Level 1; − 15 at Level 2 or 3

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| Deformation sensors | PLD, SET, INC, EXT, JNT, TLT |  |
| Structural finding | High severity, not Closed, text/component mentions crack, joint, settlement, deformation, spalling, structural |  |

### Method

Additive penalties.

### Assumptions and limits

- Open findings are taken from Inspections when the dam has any; otherwise from the manual input

### Worked example

Inputs: Rating 4; 1 open High crack finding

85 − 10

Result: 75

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §9.1 | Project requirement |

## 4.3 DHI — Seepage & leakage

Used in: DHI page   ·   Executed in system

Condition from seepage-group instruments.

### Formula

Start 100

− 10 / − 25 / − 40 per sensor at Level 1 / 2 / 3 (SPF, DRF, TRB, UPL, PWP, PHL)

− 10 if seepage flow is rising (b > 0) and r with reservoir < 0.6

− 20 if turbidity at or above its alert level

### Method

Additive penalties, clipped at 0.

### Worked example

Inputs: One SPF at Level 1; SPF rising with r = 0.45

100 − 10 − 10

Result: 80

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §9.1 | Project requirement |

## 4.4 DHI — Instrumentation

Used in: DHI page   ·   Executed in system

Health of the monitoring system itself.

### Formula

I = 0.5 × functional % + 0.3 × availability % + 0.2 × calibration-in-date %

### Method

Weighted percentages.

### Assumptions and limits

- Calibration in date = calibration_due ≥ data cut-off

### Worked example

Inputs: Functional 100 %, availability 96.7 %, calibration 83 %

50 + 29.0 + 16.6

Result: 95.6

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §9.1 | Project requirement |

## 4.5 DHI — Hydrological safety

Used in: DHI page   ·   Executed in system

Reservoir operation and flood safety.

### Formula

Start 100

− 1 per day with reservoir above FRL in last 30 days (max − 20)

− 20 if reservoir exceeded MWL in last 30 days

− 20 if 7-day trend projection exceeds FRL

− 15 per spillway gate unavailable

### Method

Additive penalties.

### Assumptions and limits

- Rule-curve compliance will be added when the rule curve is in the template

### Worked example

Inputs: 2 days above FRL; 1 gate unavailable

100 − 2 − 15

Result: 83

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §9.1 | Project requirement |

## 4.6 DHI — O&M / inspection compliance

Used in: DHI page   ·   Executed in system

How well routine obligations are met.

### Formula

O = 0.4 × inspections on time % + 0.4 × actions closed on time % + 0.2 × maintenance done %

### Method

Weighted percentages (manual inputs in v1.0).

### Worked example

Inputs: 80 %, 70 %, 85 %

32 + 28 + 17

Result: 77

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §9.1 | Project requirement |
| Dam Safety Act, 2021 | Inspection obligations |

## 4.7 DHI — Documentation

Used in: DHI page, Documents   ·   Executed in system

Availability of key approved documents.

### Formula

EAP approved 30 + O&M manual 20 + instrument register 20 + as-built drawings 15 + latest safety review 15

### Method

Points for each approved document (Documents module; manual ticks if no documents registered).

### Assumptions and limits

- Instrument register counted when sensors exist

### Worked example

Inputs: EAP revision due; safety review approved; sensors exist

0 + 0 + 20 + 0 + 15

Result: 35

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §9.1 | Project requirement |
| Dam Safety Act, 2021 | EAP and records obligations |

## 4.8 Risk score and rating

Used in: Portfolio, Overview, DHI page   ·   Executed in system

Ranks dams for attention by combining condition and consequence.

### Formula

Risk = min(100, (100 − DHI) × 1.6 × H + 4 × A + 6 × O)

Rating: Action ≥ 60 · Watch 40–59 · Monitor 25–39 · Stable < 25

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| H | Hazard factor: High 1.0, Significant 0.7, Low 0.4 | – |
| A | Strong anomalies (¦z¦ > 4) in 30 days, excluding false alarms | count |
| O | 1 if a scheduled inspection is overdue | 0/1 |

### Method

Linear index.

### Assumptions and limits

- Factors are starting values pending calibration

### Worked example

Inputs: DHI 64.8, High hazard, A = 3, O = 1

35.2 × 1.6 × 1.0 = 56.3

+ 12 + 6 = 74.3

Result: 74 → Action

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-001 §9.2 | Project requirement |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Risk score | 74 | 74 | PASS |

# 5. Design — dam

## 5.1 Wave height and freeboard

Used in: Design: Top of dam & freeboard   ·   Executed in system

Sets the top of dam above the maximum water level.

### Formula

hw = 0.032·√(V·F) + 0.763 − 0.271·F^0.25   (F < 32 km)

hw = 0.032·√(V·F)   (F ≥ 32 km)

FB = max(1.5·hw, FB_min);  FB_min = 1.5 m concrete, 2.0 m embankment

Top of dam = MWL + FB

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| V | Wind velocity | km/h |
| F | Fetch | km |
| hw | Significant wave height | m |

### Method

Molitor's empirical relation.

### Assumptions and limits

- FB_min values are screening values; embankment settlement camber added separately

### Worked example

Inputs: V = 120 km/h, F = 8 km, MWL 673 m

0.032·√960 + 0.763 − 0.271·8^0.25 = 1.299 m

FB = max(1.95, 1.5) = 1.95 m

Result: Top of dam (concrete) = 674.95 m

### References

| Document | Scope |
| --- | --- |
| IS 6512 — Criteria for design of solid gravity dams | Freeboard for concrete dams |
| IS 10635 — Freeboard requirements in embankment dams | Freeboard for embankment dams |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Molitor wave height | 1.29872 | 1.29872 | PASS |

## 5.2 Horizontal seismic coefficient (placeholder)

Used in: Design: stability, Sensitivity   ·   Executed in system

Seismic load for preliminary screening until site-specific values exist.

### Formula

αh = (Z / 2) · I

Z: zone II 0.10, III 0.16, IV 0.24, V 0.36 ;  I = 1.5

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| Z | Zone factor | – |
| I | Importance factor (assumed) | – |

### Method

Simplified seismic coefficient; response reduction and spectral shape not applied.

### Assumptions and limits

- Placeholder only — site-specific parameters (approved by NCSDP for river valley projects) govern detailed design

### Worked example

Inputs: Zone III

(0.16 / 2) × 1.5

Result: αh = 0.12

### References

| Document | Scope |
| --- | --- |
| IS 1893 (Part 1) — Criteria for earthquake resistant design | Zone factors Z |
| Site-specific seismic study | Governs final design values |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Seismic coefficient zone III | 0.12 | 0.12 | PASS |

## 5.3 Gravity dam loads (per metre run)

Used in: Design: Main dam stability   ·   Executed in system

Forces acting on the maximum section.

### Formula

W = γc · T · H + ½ · γc · (B − T) · H

U = ½ · γw · h · u · B   acting at 2B/3 from toe (triangular, reduced by drains)

P = ½ · γw · h²   at h/3

Pe = (7/12) · αh · γw · h²   at 0.4h   (Westergaard, vertical face)

Fh = αh · W   at the section centroid

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| γc, γw | Unit weight of concrete / water (9.81) | kN/m³ |
| H, h | Dam height, water depth above foundation | m |
| B, T | Base and top width | m |
| u | Uplift factor (1.0 no drains; < 1 drains working) | – |
| αh | Seismic coefficient | – |

### Method

2-D rigid body, vertical upstream face, triangular downstream face.

### Assumptions and limits

- Silt, tailwater, ice, wave and temperature loads not included in v1.0

- Drains-inoperative case uses u = 1.0

### Worked example

Inputs: H = h = 74, B = 58, T = 8, γc = 24, u = 0.67

W = 58,608 kN/m

U = 14,105 kN/m

P = 26,860 kN/m

Result: Net vertical ΣV = 44,503 kN/m

### References

| Document | Scope |
| --- | --- |
| IS 6512 — Criteria for design of solid gravity dams | Load types and combinations |

## 5.4 Sliding — shear friction

Used in: Design: Main dam stability, Sensitivity   ·   Executed in system

Checks the dam will not slide on its foundation.

### Formula

FS_sf = (μ · ΣV + c · B) / ΣH

FS_friction = μ · ΣV / ΣH (shown for information)

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| μ | Friction coefficient (tan φ) at the dam–rock contact | – |
| c | Cohesion at the contact | kPa |
| ΣV | W − U | kN/m |
| ΣH | P + Fh + Pe | kN/m |

### Method

Limit equilibrium, horizontal base.

### Assumptions and limits

- c and μ are design values; codes apply partial factors (larger on c) — confirm with the governing standard

### Acceptance criteria

- Screening limits (Rules): normal ≥ 1.5, flood ≥ 1.3, seismic ≥ 1.2, drains inoperative ≥ 1.2

### Worked example

Inputs: Same section, normal case (αh = 0)

(0.75 × 44,503 + 400 × 58) / 26,860

Result: FS = 2.11

### References

| Document | Scope |
| --- | --- |
| IS 6512 — Criteria for design of solid gravity dams | Sliding criteria and partial factors |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Sliding FS (shear friction) | 2.10639 | 2.10639 | PASS |

## 5.5 Overturning

Used in: Design: Main dam stability   ·   Executed in system

Checks the dam will not tip about its toe.

### Formula

FS_ot = ΣMr / ΣMo  (moments about the toe)

Mr = W₁·(B − T/2) + W₂·(2/3)(B − T)

Mo = P·h/3 + U·2B/3 + Fh·ȳ + Pe·0.4h

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| W₁, W₂ | Rectangular and triangular parts of the section | kN/m |
| ȳ | Height of section centroid | m |

### Method

Moment equilibrium.

### Acceptance criteria

- Screening limits: normal ≥ 1.5, seismic ≥ 1.2

### Worked example

Inputs: Same section, normal case

ΣMr = 2,247,232 kNm/m

ΣMo = 1,207,935 kNm/m

Result: FS = 1.86

### References

| Document | Scope |
| --- | --- |
| IS 6512 — Criteria for design of solid gravity dams | Overturning |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Overturning FS | 1.86039 | 1.86039 | PASS |

## 5.6 Base pressures and middle third

Used in: Design: Main dam stability   ·   Executed in system

Checks contact pressure and tension at the heel.

### Formula

x̄ = (ΣMr − ΣMo) / ΣV  (from toe)

e = B/2 − x̄

σ_toe, σ_heel = (ΣV / B) · (1 ± 6e / B)

Middle third: |e| ≤ B / 6

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| e | Eccentricity of resultant | m |

### Method

Linear base-pressure distribution.

### Acceptance criteria

- No tension at heel in the normal case (σ_heel ≥ 0)

### Worked example

Inputs: Same section, normal case

e = 5.65 m (B/6 = 9.67)

Result: σ_toe = 1,215 kPa, σ_heel = 319 kPa

### References

| Document | Scope |
| --- | --- |
| IS 6512 — Criteria for design of solid gravity dams | Stress criteria |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Toe pressure | 1,215.5 | 1,215.5 | PASS |
| Heel pressure | 319.095 | 319.095 | PASS |

## 5.7 Gravity section sizing (iteration)

Used in: Design: Main dam stability, Dam type selection   ·   Executed in system

Finds the smallest base width that passes all load cases.

### Formula

H = top of dam − foundation level ;  T = max(6, 0.14·H rounded up to 0.5 m)

B starts at 0.65·H and increases by 0.5 m until every check passes (max 1.6·H)

Cases: A normal (FRL) · C flood (MWL) · D normal + earthquake · G drains inoperative

### Method

Iterative search.

### Assumptions and limits

- Foundation level = river bed − foundation depth

### Worked example

Inputs: H 82.9 m (default project)

B tried from 54 m upward

Result: First B passing all cases is adopted

### References

| Document | Scope |
| --- | --- |
| IS 6512 — Criteria for design of solid gravity dams | Load combinations |

## 5.8 Embankment slopes — infinite-slope screening

Used in: Design: Dam type selection, Main dam stability   ·   Executed in system

Sizes embankment slopes for steady seepage, drawdown and earthquake.

### Formula

T = max(6, H/5 + 3) ≤ 15 m

Start slopes (H:V): zoned 2.75 / 2.25 ; homogeneous 3.0 / 2.5 ; CFRD 1.4 / 1.4

+0.5 soil, +0.25 fair rock, +0.25 zone IV, +0.5 zone V, +0.25 if H > 50 m (earthfill)

FS = tan φ (cos β − k sin β) / (sin β + k cos β),  tan β = 1 / n

Drawdown: FS = r · tan φ · n  (r = 0.65 zoned, 0.5 homogeneous)

Upstream seismic uses 2k (submerged); flatten failing slope by 0.25 until all pass

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| φ | Shell friction angle (CFRD rockfill 48°, homogeneous φ − 2°) | ° |
| n | Slope (horizontal per 1 vertical) | – |
| k | Seismic coefficient | – |

### Method

Cohesionless infinite slope, pseudo-static.

### Assumptions and limits

- Screening only — circular / wedge limit-equilibrium analysis required at Feasibility

### Acceptance criteria

- Steady seepage ≥ 1.5 · drawdown ≥ 1.3 · seismic ≥ 1.0

### Worked example

Inputs: φ = 34°, n = 2.5, k = 0

tan 34° × 2.5 = 0.6745 × 2.5

Result: FS = 1.69

### References

| Document | Scope |
| --- | --- |
| IS 7894 — Stability analysis of earth dams | Load cases and factors |
| IS 8826 — Design of large earth and rockfill dams | Section guidance |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Infinite-slope FS | 1.68627 | 1.68627 | PASS |

## 5.9 Dam type suitability score

Used in: Design: Dam type selection   ·   Executed in system

Ranks the four dam types for the site.

### Formula

Score = foundation pts + valley pts + seismic pts + flood pts (+ homogeneous height pts) + 20 × (lowest cost / option cost)

Not feasible → score ≤ 8 ;  failing checks → − 5

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| Gravity | Foundation E30/G25/F12/Soil 0 · valley V18/U20/W8 · zone II20/III18/IV12/V6 · flood >8000 m³/s 18 else 10 |  |
| CFRD | Foundation 26/26/16/3 · valley 20/18/10 · seismic 18 · flood 6/12 |  |
| Earthfill | Foundation 18/20/25/30 · valley 6/16/26 · zone 18/17/14/10 · flood 4/12 · homogeneous +10 if H ≤ 30 else −20 |  |

### Method

Multi-criteria scoring.

### Assumptions and limits

- Material availability and environmental impact not scored yet

### Worked example

Inputs: Excellent rock, U-valley, zone III, 12 000 m³/s

Gravity: 30 + 20 + 18 + 18 + cost points

Result: Highest score is recommended

### References

| Document | Scope |
| --- | --- |
| USBR — Design of Small Dams | Type selection practice |

## 5.10 Quantities

Used in: Design: cost, alternatives   ·   Executed in system

Volume of the main dam body.

### Formula

Section area = H · (B + T) / 2

Volume = area × crest length × valley shape factor (V 0.45, U 0.62, wide 0.80)

CFRD face slab = L·f·H·√(1 + n²) · (0.3 + 0.002·H)

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| f | Valley shape factor | – |

### Method

Prismatic approximation.

### Assumptions and limits

- Excavation, foundation treatment not included in v1.0

### Worked example

Inputs: H 82.9, B 54, T 12, L 480, U-valley

2 735 m² × 480 × 0.62

Result: ≈ 0.81 M m³

### References

| Document | Scope |
| --- | --- |
| Project quantity norms | To be replaced by 3-D model take-off |

# 6. Design — hydraulics

## 6.1 Spillway length (ogee crest)

Used in: Design: Spillway capacity & gates   ·   Executed in system

Length of gated ogee crest needed to pass the design outflow at MWL.

### Formula

He = MWL − crest level

L = Q / (C · He^1.5)

bays n = ⌈L / b⌉ ;  gross width = n·b + (n − 1)·p

gate height = (FRL − crest) + 0.5 m

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| Q | Design outflow | m³/s |
| C | Discharge coefficient (SI) | – |
| b, p | Bay width, pier width | m |

### Method

Weir equation.

### Assumptions and limits

- Pier and abutment contraction not deducted in v1.0

### Acceptance criteria

- Head on crest 3–15 m; width ≤ 70 % of crest (overflow) or ≤ 35 % of valley (chute)

### Worked example

Inputs: Q 12 000, MWL 673, crest 658, C 2.2, b 15, p 3

He = 15 m

L = 12 000 / (2.2 × 15^1.5) = 93.9 m

n = 7

Result: Gross width = 123 m

### References

| Document | Scope |
| --- | --- |
| IS 6934 — Hydraulic design of high ogee overflow spillways | Crest shape and coefficient |
| IS 11223 — Guidelines for fixing spillway capacity | Design flood |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Ogee crest length | 93.8905 | 93.8905 | PASS |
| Spillway gross width | 123 | 123 | PASS |

## 6.2 Reservoir flood routing (modified Puls)

Used in: Design: Flood routing   ·   Executed in system

Checks the maximum reservoir level and peak outflow for the inflow flood with the sized spillway.

### Formula

Storage S(h) = ∫ A(h) dh from FRL (trapezoidal, 0.01 m steps, A from the elevation–area table)

Outflow O(h) = C · (n·b) · (h − crest)^1.5

(2S/Δt + O)ₙ₊₁ = Iₙ + Iₙ₊₁ + (2S/Δt − O)ₙ

Gates regulate to hold FRL while free outflow would draw the level below FRL (then O = I)

Triangular inflow: rises 0 → Qp at tp, falls to 0 at tb; Δt = 0.5 h

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| A(h) | Reservoir area at level h | km² |
| I, O | Inflow, outflow | m³/s |
| Δt | Time step | s |

### Method

Storage-indication (modified Puls) method.

### Assumptions and limits

- Initial level = FRL

- All gates fully open once the level exceeds FRL

- Inflow hydrograph is a simplification of the design flood hydrograph

### Acceptance criteria

- Maximum level ≤ MWL ; peak outflow ≤ spillway design outflow

### Worked example

Inputs: Qp 15 000 m³/s, tp 12 h, tb 36 h, 7 bays × 15 m

Routing at 0.5 h steps

Result: Peak outflow ≈ 11 840 m³/s; max level ≈ 671.8 m

### References

| Document | Scope |
| --- | --- |
| IS 11223 — Guidelines for fixing spillway capacity | Inflow design flood and routing |
| CWC guidelines on design flood | Flood standards for large dams |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Routed peak outflow | 11,836.6 | 11,836.6 | PASS |
| Routed maximum level | 671.796 | 671.796 | PASS |
| Mass balance error < 1 % | < 1 % | < 1 % | PASS |
| Small flood held at FRL | 670 | 670 | PASS |

## 6.3 Stilling basin — hydraulic jump

Used in: Design: Energy dissipation   ·   Executed in system

Sizes the hydraulic-jump basin and checks tailwater.

### Formula

q = Q / (n · b)

V₁ = 0.9 · √(2g(E − y₁)),  y₁ = q / V₁  (iterated)

F₁ = V₁ / √(g·y₁)

y₂ = ½ · y₁ · (√(1 + 8F₁²) − 1)

ΔE = (y₂ − y₁)³ / (4·y₁·y₂) ;  L ≈ 6 (y₂ − y₁)

Tailwater depth TW = TWL_flood − floor ; if TW < y₂, floor lowered by (y₂ − TW)

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| E | Upstream energy above basin floor = MWL − floor | m |
| y₁, y₂ | Depth before and after the jump | m |
| F₁ | Froude number | – |

### Method

Energy equation with 10 % loss + Bélanger equation.

### Assumptions and limits

- Floor initially at river bed

- Slab 2 m, walls 1.5 m for indicative quantity

### Acceptance criteria

- Jump basin suited to 4.5 ≤ F₁ ≤ 9; TW ≥ y₂

### Worked example

Inputs: Q 12 000, 7 × 15 m bays, MWL 673, floor 600

q = 114.29 m²/s

y₁ = 3.437 m, V₁ = 33.25 m/s, F₁ = 5.73

Result: y₂ = 26.17 m

### References

| Document | Scope |
| --- | --- |
| IS 4997 — Criteria for design of hydraulic jump type stilling basins | Basin type and dimensions |
| USBR Engineering Monograph 25 | Stilling basin design |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Froude number F₁ | 5.72585 | 5.72585 | PASS |
| Sequent depth y₂ | 26.1678 | 26.1678 | PASS |

# 7. Design — gates & hoists

## 7.1 Radial gate hydrostatic thrust

Used in: Design: Radial gate & hoist   ·   Executed in system

Water load on each gate, passed to piers and trunnions.

### Formula

P = ½ · γw · h² · b

Trunnion reaction ≈ P / 2 per side

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| h | Water depth on gate = FRL − crest | m |
| b | Gate width | m |

### Method

Hydrostatic resultant.

### Assumptions and limits

- Hydrodynamic (seismic) load on the gate not yet included

### Worked example

Inputs: h 12 m, b 15 m

0.5 × 9.81 × 144 × 15

Result: P = 10,595 kN

### References

| Document | Scope |
| --- | --- |
| IS 4623 — Recommendations for structural design of radial gates | Gate loading |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Gate hydrostatic thrust | 10,594.8 | 10,594.8 | PASS |

## 7.2 Hoisting force and motor power

Used in: Design: Radial gate & hoist   ·   Executed in system

Size of the hoist for each gate; recalculated whenever the gate changes.

### Formula

Trunnion friction = μ · P · r / R  (μ 0.3, r 0.25 m, R = 1.25 × gate height)

F = 1.2 · (W + trunnion friction)  (20 % for seals, downpull, margin)

Motor power = F · v / η

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| W | Gate weight (input until gate design) | kN |
| v | Hoisting speed | m/s |
| η | Mechanical efficiency | – |

### Method

Force balance at start of lift.

### Assumptions and limits

- All coefficients preliminary — replace with mechanical design

### Worked example

Inputs: W 1 100 kN, h 12 m, gate height 12.5 m, v 0.3 m/min, η 0.85

friction = 0.3 × 10,595 × 0.25 / 15.62 = 51 kN

F = 1.2 × (1 100 + 51) = 1,381 kN

Result: Power = 8.1 kW per gate

### References

| Document | Scope |
| --- | --- |
| IS 6938 — Design of rope drum and chain hoists for hydraulic gates | Hoist design |
| IS 10210 — Criteria for design of hydraulic hoists | Hydraulic hoists |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Hoisting force | 1,381 | 1,381 | PASS |
| Motor power per gate | 8.12368 | 8.12368 | PASS |

# 8. Design — hydropower

## 8.1 Heads and penstock losses

Used in: Design: Hydropower chain   ·   Executed in system

Net head available to the turbine.

### Formula

Gross head range: MDDL − TWL to FRL − TWL

Rated gross head Hr = 0.67·FRL + 0.33·MDDL − TWL

V = Q / (π D² / 4)

hf = (f · L / D + Σk) · V² / 2g,  Σk = 1.5

Hn = Hr − hf

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| f | Darcy friction factor | – |
| Σk | Local loss coefficients (entrance, bends, valve) | – |

### Method

Darcy–Weisbach.

### Assumptions and limits

- Rated-head weighting is a preliminary convention

### Worked example

Inputs: Q 120, D 5.5, L 250, f 0.012

V = 5.05 m/s

hf = (0.545 + 1.5) × 1.300 = 2.66 m

Result: Head loss ≈ 2.7 m

### References

| Document | Scope |
| --- | --- |
| IS 11639 — Structural design of penstocks | Penstock practice |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Net head | 55.4404 | 55.4404 | PASS |

## 8.2 Installed capacity

Used in: Design: Hydropower chain   ·   Executed in system

Electrical power at rated conditions.

### Formula

P = ρ · g · Q · Hn · η  (W) ;  MW = 9.81 · Q · Hn · η / 1000

Units = max(2, ⌈MW / 60⌉) (indicative)

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| η | Overall efficiency (turbine × generator) | – |

### Method

Energy conversion.

### Assumptions and limits

- Annual energy needs flow-duration data (Release 2)

### Worked example

Inputs: Q 100 m³/s, Hn 50 m, η 0.90

9.81 × 100 × 50 × 0.9 / 1000

Result: 44.1 MW

### References

| Document | Scope |
| --- | --- |
| Hydropower engineering practice | Method |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Installed capacity | 58.738 | 58.738 | PASS |
| Power check 100 m³/s × 50 m | 44.145 | 44.145 | PASS |

## 8.3 Turbine type (indicative)

Used in: Design: Hydropower chain   ·   Executed in system

First-cut turbine family from net head.

### Formula

Hn > 300 m → Pelton

40 m ≤ Hn ≤ 300 m → Francis

30–40 m → Francis or Kaplan (overlap)

Hn < 30 m → Kaplan / bulb

### Method

Head-range envelope.

### Assumptions and limits

- Manufacturer application charts and specific speed govern final choice; OEM design is specialist work

### Worked example

Inputs: Hn 55.4 m

40 ≤ 55.4 ≤ 300

Result: Francis

### References

| Document | Scope |
| --- | --- |
| Turbine manufacturer application charts | Final selection |

## 8.4 Water hammer

Used in: Design: Hydropower chain   ·   Executed in system

Pressure rise when the flow is shut off.

### Formula

Critical time Tcr = 2L / a

If Tc ≤ Tcr:  Δh = a · V / g  (Joukowsky)

If Tc > Tcr:  Δh = 2 L V / (g · Tc)  (Michaud)

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| a | Pressure wave speed | m/s |
| Tc | Closure time | s |

### Method

Rigid-column / elastic approximations.

### Assumptions and limits

- Surge tank effect and governor interaction need transient analysis

### Worked example

Inputs: a 1 000 m/s, ΔV 4 m/s (instant closure)

1 000 × 4 / 9.81

Result: Δh = 408 m

### References

| Document | Scope |
| --- | --- |
| IS 11639 — Structural design of penstocks | Design pressure |
| IS 7396 — Criteria for hydraulic design of surge tanks | Surge analysis |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Water hammer (Michaud) | 25.7435 | 25.7435 | PASS |
| Joukowsky rise | 407.747 | 407.747 | PASS |

## 8.5 Penstock shell thickness

Used in: Design: Hydropower chain, cost   ·   Executed in system

Steel thickness to resist internal pressure.

### Formula

p = γw · (FRL − TWL + Δh)

t = p · D / (2 · σa · ηj) + c   (ηj 0.9, c 2 mm)

Steel = π · D · t · L × 7.85 t/m³

### Variables

| Symbol | Meaning | Unit |
| --- | --- | --- |
| σa | Allowable steel stress (input) | MPa |
| ηj | Joint efficiency | – |
| c | Corrosion allowance | mm |

### Method

Thin-shell hoop stress.

### Assumptions and limits

- External-pressure buckling, stiffeners and supports checked at Feasibility

### Worked example

Inputs: p 900 kPa, D 5.5 m, σa 150 MPa

0.9 × 5 500 / (2 × 150 × 0.9) = 18.3 mm

+ 2 mm

Result: t = 20.3 mm

### References

| Document | Scope |
| --- | --- |
| IS 11639 — Structural design of penstocks | Thickness, allowable stresses |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Penstock thickness | 20.7331 | 20.7331 | PASS |

## 8.6 Water starting time

Used in: Design: Hydropower chain   ·   Executed in system

Indicates whether a surge tank study is needed.

### Formula

Tw = L · V / (g · Hn)

### Method

Rigid water column.

### Assumptions and limits

- Indicative criterion

### Acceptance criteria

- Tw ≤ 3 s (screening) — above this, study surge relief

### Worked example

Inputs: L 250, V 5.05, Hn 55.4

250 × 5.05 / (9.81 × 55.4)

Result: Tw = 2.32 s

### References

| Document | Scope |
| --- | --- |
| IS 7396 — Criteria for hydraulic design of surge tanks | Surge requirement |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Water starting time | 2.32173 | 2.32173 | PASS |

# 9. Cost

## 9.1 Quantities, BOQ and cost build-up

Used in: Design: Quantities, BOQ & cost; Reports   ·   Executed in system

Converts design quantities to project cost.

### Formula

Line cost = quantity × rate  (quantities AUTO from design; rates user inputs)

Direct = Σ line costs

Indirect = Direct × i% ; Contingency = (Direct + Indirect) × c%

Escalation = (Direct + Indirect + Contingency) × e% ; Taxes = (… + Escalation) × t%

Total = Direct + Indirect + Contingency + Escalation + Taxes

### Method

Sequential build-up.

### Assumptions and limits

- Chute spillway concrete parametric 900 m³ per metre of width; E&M parametric per MW

### Acceptance criteria

- Preliminary accuracy ±30 %

### Worked example

Inputs: Direct ₹100 cr; 12 %, 10 %, 5 %, 12 %

100 + 12 = 112

+ 11.2 = 123.2

+ 6.16 = 129.36

+ 15.52 = 144.88

Result: Total ₹144.9 cr

### References

| Document | Scope |
| --- | --- |
| Applicable Schedule of Rates (state / CPWD / project) | Unit rates |
| Project estimating norms | Indirect and contingency |

### Validation tests

| Test | Expected | Computed | Result |
| --- | --- | --- | --- |
| Cost build-up | 144.883 | 144.883 | PASS |

# 10. Sensitivity

## 10.1 Sensitivity (tornado) analysis

Used in: Sensitivity Lab   ·   Executed in system

Shows which uncertain input moves the safety factor most.

### Formula

For each input: FS(low) and FS(high) with all else at base

Rank by range |FS(high) − FS(low)|

Conservative and worst-credible cases combine several unfavourable variations

### Method

One-at-a-time variation.

### Assumptions and limits

- Variation ranges are preset screening ranges

### Worked example

Inputs: Cohesion ±50 %

FS at c = 200 and c = 600

Result: Range plotted as a bar

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-002 §9 | Project requirement |

# 11. System rules

## 11.1 Dependency engine (STALE rule)

Used in: Design workspace   ·   Executed in system

Guarantees no result is used after its inputs change.

### Formula

When input k changes: every calculation listing k as an input becomes STALE

Then every calculation depending on a STALE calculation also becomes STALE (repeated)

STALE calculations cannot be checked or approved; re-run returns them to Draft

### Method

Directed dependency graph.

### Worked example

Inputs: FRL changed

FRL → dam type, spillway, gate, hydropower, routing

→ main dam, basin, cost

Result: 8 calculations flagged

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-002 §6 | Project requirement |

## 11.2 Check, approval and code verification

Used in: Design workspace, Users & Roles   ·   Executed in system

Engineering governance built into the software.

### Formula

Draft → Checked (Checker, not the person who ran it) → Approved (Approver, not designer or checker)

Approval blocked while any cited standard is not verified (edition and clause entered by a named engineer)

Maximum maturity in the software: Preliminary; IFC needs an authorised approver outside the prototype

### Method

Workflow with segregation of duties.

### Worked example

Inputs: Approver opens a Checked calculation citing IS 6512 (unverified)

Code check fails

Result: Approval blocked — CODE VERIFICATION REQUIRED

### References

| Document | Scope |
| --- | --- |
| FRS RCPL-DMAI-FRS-002 §7, §16 | Project requirement |

# 12. Validation record

The software includes an automatic validation suite of 37 tests. Each test feeds known inputs into the same function the software uses and compares the output with an independently computed value (hand calculation or a separate Python implementation). Result of the release run: 37 passed, 0 failed.

| # | Test | Formula | Inputs | Expected | Computed | Tol. | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | z-score of a reading | Anomaly detection — rolling z-score | window 10,12,11,13,12,11,10,12,13,11; v=16 | 4.1662 | 4.16619 | ± 0.0001 | PASS |
| 2 | Least-squares slope | Trend (least-squares slope) | x 0..4; y 10,12,13,15,16 | 1.5 | 1.5 | ± 1e-9 | PASS |
| 3 | Pearson correlation | Correlation with reservoir level | a 1..5; b 2,4,5,4,5 | 0.774597 | 0.774597 | ± 0.00001 | PASS |
| 4 | Days to alert | Days to alert level | alert 290, ŷ₀ 272.3, b 0.89/day | 20 | 20 | exact | PASS |
| 5 | Triage — near alert | Anomaly triage | rising, alert 215, v 206, z 7 | Near alert level | Near alert level | exact | PASS |
| 6 | Triage — strong anomaly | Anomaly triage | rising, alert 215, v 150, z 4.5 | Strong anomaly | Strong anomaly | exact | PASS |
| 7 | Threshold level | Threshold level of the latest reading | alert 290, alarm 340, emergency 420, v 295.2 | 1 | 1 | exact | PASS |
| 8 | Flatline flags 14 identical readings | QC-03 Flatline | 20 varying + 14 × 9.82 | 14 | 14 | exact | PASS |
| 9 | Isolated spike flagged once | QC-02 Isolated spike | 40 readings ±0.5 with one +25 jump | 1 | 1 | exact | PASS |
| 10 | DHI weighted sum | Dam Health Index (DHI) | S66 Sp50 I70 H74 O60 D77 | 64.8 | 64.8 | ± 1e-9 | PASS |
| 11 | Risk score | Risk score and rating | DHI 64.8, High, 3 strong, overdue | 74 | 74 | exact | PASS |
| 12 | Molitor wave height | Wave height and freeboard | V 120 km/h, F 8 km | 1.29872 | 1.29872 | ± 0.0001 | PASS |
| 13 | Seismic coefficient zone III | Horizontal seismic coefficient (placeholder) | Z 0.16, I 1.5 | 0.12 | 0.12 | ± 1e-9 | PASS |
| 14 | Sliding FS (shear friction) | Sliding — shear friction | H=h 74, B 58, T 8, γc 24, μ .75, c 400, u .67 | 2.10639 | 2.10639 | ± 0.0001 | PASS |
| 15 | Overturning FS | Overturning | same section | 1.86039 | 1.86039 | ± 0.0001 | PASS |
| 16 | Toe pressure | Base pressures and middle third | same section | 1,215.5 | 1,215.5 | ± 0.01 | PASS |
| 17 | Heel pressure | Base pressures and middle third | same section | 319.095 | 319.095 | ± 0.01 | PASS |
| 18 | Infinite-slope FS | Embankment slopes — infinite-slope screening | φ 34°, n 2.5, k 0 | 1.68627 | 1.68627 | ± 1e-9 | PASS |
| 19 | Ogee crest length | Spillway length (ogee crest) | Q 12000, He 15, C 2.2 | 93.8905 | 93.8905 | ± 0.001 | PASS |
| 20 | Spillway gross width | Spillway length (ogee crest) | 7 bays × 15 + 6 × 3 | 123 | 123 | exact | PASS |
| 21 | Froude number F₁ | Stilling basin — hydraulic jump | Q 12000, 7×15 m, MWL 673, floor 600 | 5.72585 | 5.72585 | ± 0.001 | PASS |
| 22 | Sequent depth y₂ | Stilling basin — hydraulic jump | same | 26.1678 | 26.1678 | ± 0.001 | PASS |
| 23 | Routed peak outflow | Reservoir flood routing (modified Puls) | Qp 15000, tp 12 h, tb 36 h, 7×15 m, default table | 11,836.6 | 11,836.6 | ± 118.366334147975 | PASS |
| 24 | Routed maximum level | Reservoir flood routing (modified Puls) | same | 671.796 | 671.796 | ± 0.05 | PASS |
| 25 | Mass balance error < 1 % | Reservoir flood routing (modified Puls) | same | < 1 % | < 1 % | exact | PASS |
| 26 | Small flood held at FRL | Reservoir flood routing (modified Puls) | Qp 5000 (below capacity at FRL) | 670 | 670 | ± 0.000001 | PASS |
| 27 | Gate hydrostatic thrust | Radial gate hydrostatic thrust | h 12, b 15 | 10,594.8 | 10,594.8 | ± 0.01 | PASS |
| 28 | Hoisting force | Hoisting force and motor power | W 1100 kN, gate height 12.5 | 1,381 | 1,381 | ± 0.01 | PASS |
| 29 | Motor power per gate | Hoisting force and motor power | v 0.3 m/min, η 0.85 | 8.12368 | 8.12368 | ± 0.001 | PASS |
| 30 | Net head | Heads and penstock losses | Q 120, D 5.5, L 250, f .012, FRL 670, MDDL 640, TWL 602 | 55.4404 | 55.4404 | ± 0.001 | PASS |
| 31 | Installed capacity | Installed capacity | same, η 0.9 | 58.738 | 58.738 | ± 0.001 | PASS |
| 32 | Power check 100 m³/s × 50 m | Installed capacity | P = 9.81·100·50·0.9/1000 | 44.145 | 44.145 | ± 0.001 | PASS |
| 33 | Water hammer (Michaud) | Water hammer | L 250, V 5.05, Tc 10 s | 25.7435 | 25.7435 | ± 0.001 | PASS |
| 34 | Joukowsky rise | Water hammer | a 1000, ΔV 4 (Tc 0.1 s) | 407.747 | 407.747 | ± 0.001 | PASS |
| 35 | Penstock thickness | Penstock shell thickness | σa 150 MPa, ηj 0.9, c 2 mm | 20.7331 | 20.7331 | ± 0.001 | PASS |
| 36 | Water starting time | Water starting time | L 250, V 5.05, Hn 55.4 | 2.32173 | 2.32173 | ± 0.0001 | PASS |
| 37 | Cost build-up | Quantities, BOQ and cost build-up | Direct ₹100 cr; 12/10/5/12 % | 144.883 | 144.883 | ± 0.0001 | PASS |

# 13. Reference register

Standards and guidance cited by the software. Titles are as commonly cited; confirm against the official publication. Edition and clause verification is recorded in the software by a named engineer.

| Reference | Edition / year | Clauses relied on | Verified by / date |
| --- | --- | --- | --- |
| IS 6512 — Criteria for design of solid gravity dams |  |  |  |
| IS 1893 (Part 1) — Criteria for earthquake resistant design of structures |  |  |  |
| IS 7894 — Code of practice for stability analysis of earth dams |  |  |  |
| IS 8826 — Guidelines for design of large earth and rockfill dams |  |  |  |
| IS 10635 — Guidelines for freeboard requirements in embankment dams |  |  |  |
| IS 11223 — Guidelines for fixing spillway capacity |  |  |  |
| IS 6934 — Recommendations for hydraulic design of high ogee overflow spillways |  |  |  |
| IS 4997 — Criteria for design of hydraulic jump type stilling basins |  |  |  |
| IS 4623 — Recommendations for structural design of radial gates |  |  |  |
| IS 6938 — Design of rope drum and chain hoists for hydraulic gates |  |  |  |
| IS 10210 — Criteria for design of hydraulic hoists for gates |  |  |  |
| IS 11639 — Structural design of penstocks — criteria |  |  |  |
| IS 7396 — Criteria for hydraulic design of surge tanks |  |  |  |
| IS 7436 — Guide for types of measurements for structures in river valley projects |  |  |  |
| USBR — Design of Small Dams |  |  |  |
| USBR Engineering Monograph 25 — Hydraulic design of stilling basins and energy dissipators |  |  |  |
| Dam Safety Act, 2021 (Government of India) |  |  |  |
| CWC guidelines on design flood for dams |  |  |  |
| Applicable Schedule of Rates (state / CPWD / project) |  |  |  |